# Post Lab Questions - Series Circuit

# 5. Role of resistor in series circuit

A resistor limits the flow of current, divides voltage, and protects components by controlling current in the circuit.

# 6. Voltage Divider Rule

Voltage divider rule states that in a series circuit, the voltage across a resistor is proportional to its resistance. Vx = (Rx / Rtotal) × Vtotal

# 7. Features of Series Circuit

- Same current flows through all components.
- Total resistance is the sum of all resistors.
- Voltage is divided among resistors.
- If one component fails, entire circuit stops.
- Simple and easy to analyze.

# 8. Difference between Voltmeter and Ammeter

Voltmeter:
- Connected in parallel
- High internal resistance
- Measures voltage

Ammeter:
- Connected in series
- Low internal resistance
- Measures current

# 9. DMM Connection

To measure current, DMM is connected in series.
To measure voltage, DMM is connected in parallel.